University of Nevada Las Vegas. Department of Electrical and Computer Engineering Laboratories.

| Class: | CPE300L - Digital System Architecture and Design Lab |  |  | Semester: | Fall 2025 |
| --- | --- | --- | --- | --- | --- |
| Points |  | Document author: | Darryll Mckoy |  |  |
|  |  | Author's email: | mckoyd1@unlv.nevada.edu |  |  |
|  |  | Document topic: | Postlab 7 |  |  |
| Instructor's comments: |  |  |  |  |  |

- Lab submissions

This lab took around 7 hours

2.  Marked Figure 1:

- Marked Figure 2:

Marked Figure 3:

- Amended Tables

Main Decoder Truth Table:

| Instruction | Opcode | RegWrite | ImmSrc | ALUSrcA | ALUSrcB | MemWrite | ResultSrc | Branch | ALUOp | Jump |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw | 0000011 | 1 | 000 | 0 | 1 | 0 | 01 | 0 | 00 | 0 |
| sw | 0100011 | 0 | 001 | 0 | 1 | 1 | 00 | 0 | 00 | 0 |
| R-type | 0110011 | 1 | xxx | 0 | 0 | 0 | 00 | 0 | 10 | 0 |
| beq | 1100011 | 0 | 010 | 0 | 0 | 0 | 00 | 1 | 01 | 0 |
| I-type ALU | 0010011 | 1 | 000 | 0 | 1 | 0 | 00 | 0 | 10 | 0 |
| jal | 1101111 | 1 | 011 | 0 | 0 | 0 | 10 | 0 | 00 | 1 |
| lui | 0110111 | 1 | 100 | 1 | 1 | 0 | 00 | 0 | 00 | 0 |

ALU Decoder Truth Table:

| ALUOp1:0 | funct32:0 | {op5, funct75} | ALUControl2:0 | Operation |
| --- | --- | --- | --- | --- |
| 00 | x | x | 000 | Add |
| 01 | x | x | 001 | Subtract |
| 10 | 000 | 00, 01, 10 | 000 | Add |
|  | 000 | 11 | 001 | Subtract |
|  | 010 | x | 101 | SLT |
|  | 110 | x | 011 | OR |
|  | 111 | x | 010 | AND |
|  | 100 | x | 100 | XOR |

ImmSrc Truth Table:

| ImmSrc | ImmExt | Type | Description |
| --- | --- | --- | --- |
| 000 | {{20{Instr[31]}}, Instr[31:20]} | I | 12-bit signed immediate |
| 001 | {{20{Instr[31]}}, Instr[31:25], Instr[11:7]} | S | 12-bit signed immediate |
| 010 | {{20{Instr[31]}}, Instr[7], Instr[30:25], Instr[11:8], 1’b0} | B | 12-bit signed immediate |
| 011 | {{12{Instr[31]}}, Instr[19:12], Instr[20], Instr[30:21], 1’b0} | J | 12-bit signed immediate |
| 100 | {Instr[31:12], 12’b0} | U | 20-bit signed immediate |

- Amended System Verilog Code For Lui:

- Made 4 modifications to riscvsingle module, adding ALUsrcA/B, increased ImmSrc to 3 bits.

- Increased ImmSrc to 3 bits and added ALUsrcA/B within controller module.

- Increased ImmSrc to 3 bits, increased and updated controls to 13 bits, and updated all control values with addition of lui. Added ALUSrcA/B to maindec module.

- Modified datapath to include ALUSrcA/B, increased ImmSrc to 3 bits, addeed rd1 logic, added mux for lui.

- Added lui immext, increased immsrc to 3 bits.

Amended System Verilog Code For XOR:

- Added XOR function to aludec module.

Ammended System Verilog Code for DE2 Board

- Modified top module to include 7-seg decoder to display instruction, moved PC/Instr to output logic.

- Modified Assembly Instruction:

- Simulation Waveform:

- DE2 Board Screenshot and Demo

Demo Link:  https://youtu.be/LvzOv1TrRXI?si=6XC6et_URz_6zXoA